# CLAUDE.md

This file provides guidance to Claude Code (claude.ai/code) when working with code in this repository.

## Project Overview

FEM-JAX-GEO is a differentiable geomechanics finite element package built on top of JAX-FEM. It implements constitutive models (primarily Drucker-Prager plasticity) for gradient-based optimization in geomechanics using automatic differentiation.

The project structure includes:

Custom constitutive models built on JAX-FEM's Problem class

Full FEM simulations with path-dependent material behavior

Differentiability testing via AD vs FD comparison

Post-processing utilities for comparing analytical vs numerical results

## JAX-FEM Integration

This project relies on the JAX-FEM library located in jax-fem-main/. JAX-FEM is a differentiable finite element package based on JAX that provides:

AD + FEM capabilities with GPU acceleration

2D/3D elements (quad, triangle, hex, tetrahedron)

First and second order elements

Linear and nonlinear analysis (heat, elasticity, hyperelasticity, plasticity)

Multi-physics support

Integration with PETSc for solver options

### Key JAX-FEM Classes

**Problem** (jax_fem/problem.py): Base class for defining FEM problems. Handles one or more coupled FE variables, mesh management, and boundary conditions.

**FiniteElement** (jax_fem/fe.py): Core FE implementation managing mesh, shape functions, quadrature.

**Mesh** (jax_fem/generate_mesh.py): Mesh data structure and generation utilities.

**solver** (jax_fem/solver.py): Nonlinear solver with multiple linear solver backends (JAX, scipy UMFPACK, PETSc).

## Architecture

### Constitutive Model Implementation Pattern

Custom material models in src/models/ inherit from jax_fem.problem.Problem:

Initialization: Set material parameters (E, nu, alpha, k, etc.) and pass mesh/BC info to Problem

custom_init(): Initialize internal variables (stress/strain history arrays)

get_tensor_map(): Return the stress computation function for the solver

get_maps(): Define the core constitutive relations:

strain(u_grad): Compute strain from displacement gradient

stress_return_map(u_grad, sigma_old, epsilon_old): Compute stress using return mapping algorithm

update_stress_strain(sol): Update internal variables after each load step

Helper methods: Problem-specific utilities like compute_avg_stress()

The stress return mapping must handle:

Elastic trial stress

Yield function evaluation

Plastic correction (return to yield surface)

Safe mathematical operations (avoiding division by zero, negative square roots)

### Path Setup Pattern

Scripts use this pattern to ensure jax-fem-main is in the Python path:

project_root = os.path.abspath(os.path.join(os.path.dirname(__file__), '..'))

jax_fem_path = os.path.join(project_root, 'jax-fem-main')

if jax_fem_path not in sys.path:

sys.path.append(jax_fem_path)

## Common Development Commands

### Running Simulations

# Full FEM simulation of 3D block under compression

python examples/run_fem_simulation.py

# Single-point constitutive verification driver

python examples/run_constitutive_driver.py

### Running Tests

Tests verify differentiability (AD vs FD) for various constitutive model implementations:

# Run specific test

python tests/test_diff_dp.py                    # Basic Drucker-Prager differentiability

python tests/test_diff_elastic.py               # Elastic model

python tests/test_diff_dp_strategy1_visco.py    # Viscoplastic regularization

python tests/test_diff_dp_strategy2_direct.py   # Direct differentiation

python tests/test_diff_dp_strategy3_smooth.py   # Smooth yield surface

python tests/test_diff_dp_hardening.py          # Hardening model

python tests/test_diff_dp_bc_fix.py             # BC handling

# Run all tests in the directory

python -m pytest tests/

### Post-Processing

# Compare FEM results with analytical solutions

python scripts/compare_results.py

## Boundary Condition Patterns

JAX-FEM uses a three-element list structure for Dirichlet BCs:

dirichlet_bc_info = [location_fns, vecs, value_fns]

Where:

location_fns: List of callables that identify BC locations (return bool for each point)

vecs: List of component indices (0=x, 1=y, 2=z for 3D problems)

value_fns: List of callables that return the prescribed value

Example for compression test:

def bottom(point): return np.isclose(point[2], 0., atol=1e-5)

def top(point): return np.isclose(point[2], Lz, atol=1e-5)

location_fns = [bottom, top]

vecs = [2, 2]  # z-component for both

value_fns = [lambda p: 0., lambda p: displacement]

## Solver Configuration

The solver() function accepts options for different linear solver backends:

# PETSc solver (recommended for large problems)

solver_options = {'petsc_solver': {}}

sol_list = solver(problem, solver_options=solver_options)

# JAX solver (iterative, GPU-compatible)

solver_options = {'jax_solver': {}}

# Scipy UMFPACK (direct solver)

solver_options = {'umfpack_solver': {}}

## Incremental Loading Pattern

For path-dependent materials, simulations use incremental loading:

Define displacement history (load/unload cycles)

Loop over load steps

Update boundary conditions for current step

Solve equilibrium equations

Update internal variables (stress/strain history)

Save results (VTK files, average stress)

## Output Organization

Results are typically saved in results/YYYY-MM-DD_description/:

vtk/: VTK files for visualization (one per load step)

fem_data.csv: Stress-strain or other tabular data

*.png: Plots generated during post-processing

## JAX-FEM Application Examples

The jax-fem-main/applications/ directory contains reference implementations:

stokes/: Stokes flow (multi-variable problem)

wave/: Wave equation (time-dependent)

crystal_plasticity/: Crystal plasticity models

phase_field_fracture/: Phase field fracture mechanics

thermal_mechanical/: Coupled thermomechanical analysis

periodic_bc/: Periodic boundary conditions

These can serve as templates for new problem types.